Documentation group 7

Aslam Jaffer

S5 ECE

Raspberry Pi Workshop – FIOT Lab

Day 1

Class started with setting up the Raspberry Pi. The SD card was flashed  before it was given to us.

We got Raspberry Pi 4 model and connected it to the computer using USB with Real VNC. We got the  username and password.

we pinged the Raspberry Pi. Enabled SSH. Then connected it to VNC.

After this, we tried LED blinking.

Code – LED Blinking

import RPi.GPIO as GPIO

import time

LED_PIN = 17

GPIO.setmode(GPIO.BCM)

GPIO.setup(LED_PIN, GPIO.OUT)

for i in range(5):

GPIO.output(LED_PIN, True)

time.sleep(1)

GPIO.output(LED_PIN, False)

time.sleep(1)

GPIO.cleanup()

Day 2 –

On the second day, we moved on to interfacing sensors. The first was a touch sensor. We connected the touch sensor to the Raspberry Pi GPIO pins.

Code – Touch Sensor with LED

import RPi.GPIO as GPIO

import time

TOUCH_PIN = 18

LED_PIN = 17

GPIO.setmode(GPIO.BCM)

GPIO.setup(TOUCH_PIN, GPIO.IN)

GPIO.setup(LED_PIN, GPIO.OUT)

while True:

if GPIO.input(TOUCH_PIN) == 1:

GPIO.output(LED_PIN, True)

else:

GPIO.output(LED_PIN, False)

time.sleep(0.1)

except KeyboardInterrupt:

GPIO.cleanup()

Day 3 – 25/09

Class started off with a  recap of the previous day.

Stepper Motor

Unipolar stepper motor

Driver module

DC power adaptor jack

Jumper wires

The GPIO pin connections were:

IN1 - GPIO 17

IN2 - GPIO 18

IN3 - GPIO 27

IN4 – GPIO 22

GND →  Pi pin 6

The stepper motor started rotating.

Servo Motor

Yellow → GPIO pin

Red → VCC

Black → GND

Installed Tkinter to make a simple UI for controlling servo motor:

sudo apt update

sudo apt install python3-tk -y

Created a file with Tkinter code and tested basic UI buttons.

Installed pygame using:

sudo apt install python3-pygame

We tried a variety of pygames using raspberry pi.

Programs done :

Babybird.py

Import pygame

Import RPi.GPIO as GPIO

Import time

Import random

Import os

TOUCH_PIN = 18

GPIO.setmode(GPIO.BCM)

GPIO.setup(TOUCH_PIN, GPIO.IN)

Pygame.init()

WIDTH, HEIGHT = 800, 600

Screen = pygame.display.set_mode((WIDTH, HEIGHT))

Pygame.display.set_caption(“Shoot the Bird”)

Clock = pygame.time.Clock()

Bird_img = pygame.image.load(“/home/pi/Desktop/game/bird.png”).convert_alpha()

Bird_img = pygame.transform.scale(bird_img, (60, 60))  # Resize to 60x60

Bird_rect = bird_img.get_rect()

Bird_x = -bird_rect.width

Bird_y = random.randint(50, HEIGHT – 200)

Base_speed = 3

Bird_speed = base_speed

Shot_active = False

Shot_x = 0

Shot_y = HEIGHT – 50

Shot_radius = 10  # Bigger ball

Shot_speed = 10

Score = 0

Font = pygame.font.SysFont(None, 36)

Def reset_bird():

Global bird_x, bird_y

Bird_x = -bird_rect.width

Bird_y = random.randint(50, HEIGHT – 200)

Def draw_bird():

Screen.blit(bird_img, (bird_x, bird_y))

Def draw_shot(x, y):

Pygame.draw.circle(screen, (255, 0, 0), (x, y), shot_radius)

Def is_hit(shot_x, shot_y, bird_x, bird_y):

Br = bird_rect.move(bird_x, bird_y)

Return br.collidepoint(shot_x, shot_y)

Running = True

Start_time = time.time()

While running:

Screen.fill((255, 255, 255))  # white background

For event in pygame.event.get():

If event.type == pygame.QUIT:

Running = False

Elapsed = time.time() – start_time

Bird_speed = base_speed + (elapsed / 10)

Bird_x += bird_speed

If bird_x > WIDTH:

Reset_bird()

If GPIO.input(TOUCH_PIN) == GPIO.HIGH and not shot_active:

Shot_active = True

Shot_x = WIDTH // 2

Shot_y = HEIGHT – 50

if shot_active:

Draw_shot(shot_x, shot_y)

Shot_y -= shot_speed

If shot_y < 0:

Shot_active = False

Elif is_hit(shot_x, shot_y, bird_x, bird_y):

Score += 1

Shot_active = False

Reset_bird()

Draw_bird()

Score_text = font.render(f”Score: {score}”, True, (0, 0, 0))

Screen.blit(score_text, (10, 10))

Pygame.display.flip()

Clock.tick(60)

GPIO.cleanup()

pygame.quit

Servo.py

Import RPi.GPIO as GPIO

Import time

# Use BCM numbering

Servo_pin = 17  # GPIO17 (pin 11)

GPIO.setmode(GPIO.BCM)

GPIO.setup(servo_pin, GPIO.OUT)

# Set frequency to 50Hz for servo

Pwm = GPIO.PWM(servo_pin, 50)

Pwm.start(0)

Def set_angle(angle):

Duty = 2 + (angle / 18)  # Convert angle to duty cycle

GPIO.output(servo_pin, True)

Pwm.ChangeDutyCycle(duty)

Time.sleep(0.5)

GPIO.output(servo_pin, False)

Pwm.ChangeDutyCycle(0)

Try:

While True:

Set_angle(0)

Time.sleep(1)

Set_angle(90)

Time.sleep(1)

Set_angle(180)

Time.sleep(1)

Except KeyboardInterrupt:

Pwm.stop()

GPIO.cleanup()

Step.py

Import RPi.GPIO as GPIO

Import time

# Define GPIO pins

IN1 = 17

IN2 = 18

IN3 = 27

IN4 = 22

# Setup

GPIO.setmode(GPIO.BCM)

GPIO.setup(IN1, GPIO.OUT)

GPIO.setup(IN2, GPIO.OUT)

GPIO.setup(IN3, GPIO.OUT)

GPIO.setup(IN4, GPIO.OUT)

# Half-step sequence for 28BYJ-48

Step_sequence = [

[1,0,0,1],

[1,0,0,0],

[1,1,0,0],

[0,1,0,0],

[0,1,1,0],

[0,0,1,0],

[0,0,1,1],

[0,0,0,1],

]

Def set_step(pins):

GPIO.output(IN1, pins[0])

GPIO.output(IN2, pins[1])

GPIO.output(IN3, pins[2])

GPIO.output(IN4, pins[3])

Try:

Print(“Rotating stepper motor for 10 seconds...”)

Start_time = time.time()

Step = 0

While time.time() – start_time < 10:

Set_step(step_sequence[step % 8])

Step += 1

Time.sleep(0.002)  # adjust speed

Print(“Done.”)

Except KeyboardInterrupt:

Pass

Finally:

GPIO.cleanup()

Stepper. Py

Import RPi.GPIO as GPIO

Import time

# Define GPIO pins

IN1 = 17

IN2 = 18

IN3 = 27

IN4 = 22

# Setup

GPIO.setmode(GPIO.BCM)

GPIO.setup(IN1, GPIO.OUT)

GPIO.setup(IN2, GPIO.OUT)

GPIO.setup(IN3, GPIO.OUT)

GPIO.setup(IN4, GPIO.OUT)

# Define step sequence for 28BYJ-48

Step_sequence = [

[1, 0, 0, 1],

[1, 0, 0, 0],

[1, 1, 0, 0],

[0, 1, 0, 0],

[0, 1, 1, 0],

[0, 0, 1, 0],

[0, 0, 1, 1],

[0, 0, 0, 1],

]

Def set_step(pins):

GPIO.output(IN1, pins[0])

GPIO.output(IN2, pins[1])

GPIO.output(IN3, pins[2])

GPIO.output(IN4, pins[3])

Try:

Print(“Rotating stepper motor...”)

For i in range(512):  # 512 steps = approx one full revolution for 28BYJ-48

Set_step(step_sequence[i % 8])

Time.sleep(0.002)

Print(“Done.”)

Except KeyboardInterrupt:

Pass

Finally:

GPIO.cleanup()

Tinker.py

Import tkinter as tk

Import RPi.GPIO as GPIO

Import time

# GPIO pins for stepper motor

IN1 = 17

IN2 = 18

IN3 = 27

IN4 = 22

Pins = [IN1, IN2, IN3, IN4]

# Setup GPIO

GPIO.setmode(GPIO.BCM)

For pin in pins:

GPIO.setup(pin, GPIO.OUT)

GPIO.output(pin, 0)

# Half-step sequence

Step_sequence = [

[1,0,0,1],

[1,0,0,0],

[1,1,0,0],

[0,1,0,0],

[0,1,1,0],

[0,0,1,0],

[0,0,1,1],

[0,0,0,1],

]

# Step the motor

Def rotate_motor(direction):

Steps = 512  # ~1 full rotation for 28BYJ-48

Print(f”Rotating {direction}...”)

For i in range(steps):

If direction == “cw”:

Seq = step_sequence[i % 8]

Else:  # ccw

Seq = step_sequence[(7 – (i % 8))]

For pin, val in zip(pins, seq):

GPIO.output(pin, val)

Time.sleep(0.002)

# Turn off all coils

For pin in pins:

GPIO.output(pin, 0)

# GUI setup

Root = tk.Tk()

Root.title(“Stepper Motor Control”)

Frame = tk.Frame(root)

Frame.pack(padx=20, pady=20)

Btn_cw = tk.Button(frame, text=”Rotate Clockwise”, command=lambda: rotate_motor(“cw”))

Btn_cw.grid(row=0, column=0, padx=10, pady=10)

Btn_ccw = tk.Button(frame, text=”Rotate Anti-Clockwise”, command=lambda: rotate_motor(“ccw”))

Btn_ccw.grid(row=0, column=1, padx=10, pady=10)

# Graceful exit

Def on_close():

GPIO.cleanup()

Root.destroy()

Root.protocol(“WM_DELETE_WINDOW”, on_close)

Root.mainloop()

Touch. py

Import RPi.GPIO as GPIO

Import time

# GPIO pin numbers

SERVO_PIN = 17     # GPIO17 for servo

TOUCH_PIN = 18     # GPIO18 for touch sensor

# Setup

GPIO.setmode(GPIO.BCM)

GPIO.setup(SERVO_PIN, GPIO.OUT)

GPIO.setup(TOUCH_PIN, GPIO.IN)

# Initialize PWM for servo (50Hz)

Pwm = GPIO.PWM(SERVO_PIN, 50)

Pwm.start(0)

Def set_angle(angle):

Duty = 2 + (angle / 18)

GPIO.output(SERVO_PIN, True)

Pwm.ChangeDutyCycle(duty)

Time.sleep(0.5)

GPIO.output(SERVO_PIN, False)

Pwm.ChangeDutyCycle(0)

Try:

Angle = 0

While True:

If GPIO.input(TOUCH_PIN) == GPIO.HIGH:

Print(“Touch detected!”)

Angle += 90

If angle > 180:

Angle = 0

Set_angle(angle)

Time.sleep(0.5)  # debounce delay

Except KeyboardInterrupt:

Pwm.stop()

GPIO.cleanup()